Nylon-Aramid Hybrid Cords — Design, Reverse-Engineering & Performance

Zero-degree belt / cap-ply cords in PCR tires · 100 strict-cohort patents (31 with a wear claim) · audited, motorcycle-excluded

1.  What this cord is, and why

The Nylon-Aramid hybrid is a zero-degree cap-ply cord that pairs a low-shrink, high-modulus aramid filament with a flexible nylon-6.6 filament to give a two-stage (“J-shaped”) load–elongation response. Nylon carries the low-load region (flexibility, fatigue life, and post-cure shrink that tightens the cap ply during cure); aramid carries the high-load region (crown stiffness retained at speed and temperature). The design intent is high-speed durability and crown control — wear/mileage is a secondary, indirect benefit via footprint uniformity, not an abrasion-chemistry effect.

2.  Canonical construction (reverse-engineering datasheet)

Construction is consistent across assignees. A typical teardown shows:

| Parameter | Typical value(s) in the corpus |
| --- | --- |
| Aramid linear density | 1670 / 1400 / 1100 dtex (load-bearing filament) |
| Nylon linear density | 940 dtex is the signature value; nylon-6.6 in ~half the cohort |
| Ply / construction code | mostly 2-ply — e.g. 1100/2, 1670/2, 1840/2 (some single-end) |
| Twist | 200–500 TPM (peaks at 200, 300, 500), balanced ply-twist |
| Cord density | 50–90 ends/dm (EPDM), centred ~80, spiral-wound cap |
| Dual-modulus step | modulus ratio ~1.5 — the defining two-stage signature |
| Assembly style | co-twisted / cabled together dominates (43 patents); true core-sheath is the minority — and is nylon-core / aramid-sheath more often than the reverse |

Reverse-engineering tells: aramid is almost never branded (5% name Kevlar/Twaron/Technora) — a generic, low-heat-shrink high-modulus filament co-twisted with a 940-dtex PA66 filament at ~300 TPM is the textbook hybrid. Dimensional-stability / heat-set terms appear in 43% of patents (shrinkage ~0.5–4%); the aramid's low heat-shrink and the nylon's high shrink are the fingerprints in a cross-section.

3.  Performance levers and the failure mode to manage

Distinctive wear levers vs. an all-PCR baseline are in the cord itself: core-sheath/wrap (2.0×), dual-modulus (1.9×) and twist tuning (1.8×). Adhesion/dip and zoned density are NOT distinctive to Nylon-Aramid.

The aramid stiffness step creates an uneven-wear risk at the cord's modulus transition. This is why the cohort leans defensive (fixing uneven wear) and why twist tuning and modulus grading recur — they exist largely to engineer that transition out. Pre-empt this failure mode in any new design.

4.  IP landscape — design freedom & who to read

Legal status: 74 DEAD / 26 ALIVE. The large expired body is effectively a free-to-practice reverse-engineering library — the canonical recipe above is largely off-patent.

Live art (26 patents, 8 with a wear claim) clusters 2018–2025 and is the FTO-relevant set — recent holders include Goodyear, Yokohama, Sumitomo, Pirelli and Hyosung. The wear-specific angle is thin and mostly dead: an opportunity, but validate against live PET/PA “center-vs-edge” zoned-density art.

Where the know-how sits: Bridgestone Corp (16), The Goodyear Tire & Rubber Co (12), Yokohama Rubber Co Ltd (11), Michelin Research & Technology Co Ltd (9), Sumitomo Rubber Ind Ltd (8), Continental Reifen Deutschland Gmbh (5). For the yarn/cord engineering specifically, read the cord-makers Kordsa and Hyosung.

5.  Practical takeaways

Specifying a cord: start from aramid ~1670 dtex + nylon-6.6 940 dtex, 2-ply, ~300 TPM, ~80 ends/dm spiral cap, ~1.5× modulus step; then tune twist/grading to suppress the uneven-wear transition.

Reverse-engineering a competitor: the co-twist vs. nylon-core/aramid-sheath distinction, the 940-dtex nylon tell, and the aramid low-heat-shrink signature are your fingerprints.

Positioning performance: sell high-speed / high-temperature durability and crown stiffness; treat mileage as a derived benefit; pre-empt the uneven-wear failure mode.

Method note: figures are text-mined from PatSeer fields + AI summaries over the strict Nylon-Aramid cohort (n=100); high recall — spot-check claims before quoting an individual patent. Corpus skews pre-2015; legal status as of the export date.